SYSTEM ID REPORT

## 1. System Description

The selected system is a 4-wheel skid-steering rover. It is commonly used in robotic applications such as planetary exploration, surveillance, and rough terrain navigation.

2. Real-World Application

This type of rover is used in environments where traditional steering is difficult, such as Mars rovers, military robots, and autonomous ground vehicles operating on uneven terrain.

3. Inputs, Outputs, States, Parameters

Inputs:

- Mz→ yaw moment (steering input via differential torque)

Outputs:

- γ→ yaw rate

- α→ slip angle

- x,y→ position of the rover

State Variables:

- α→ slip angle

- γ→ yaw rate

Parameters:

- m→ mass of rover

- Iz→ moment of inertia

- v→ forward velocity

- a,b→ geometry distances

- Kf,Kr→ cornering stiffness

## 4. Degrees of Freedom (DOF)

The system has 2 degrees of freedom:

- Lateral motion (slip angle α)

- Rotational motion (yaw rate γ)

### Justification:

### The rover's motion is constrained to a plane, and its forward velocity is assumed constant. Hence, only lateral and rotational dynamics are considered.

## 5. Key Assumptions

- The rover moves on a flat surface

- Forward velocity vis constant

- Tire behavior follows a linear model (small slip angles)

- No vertical motion or suspension effects considered

- Air resistance is neglected

### Reasoning:

These assumptions simplify the system while preserving the dominant dynamics affecting steering and motion.

### 6. Free Body Diagram (FBD)

7. Coordinate System & Sign Convention

- x-axis → forward direction of rover

- y-axis → lateral direction

- Positive γ→ counterclockwise rotation

- Positive α→ deviation to the left