# Project Title - Digital Twin of Multi-Motor System with Self-Healing Capability

# Category E — Digital Twins / Electromechanical Systems

# Real-World Application

This system models industrial multi-motor setups used in conveyor belts, robotics, and manufacturing lines, where fault detection and self-healing ensure continuous operation and prevent system failure.

# System Description

The system consists of three different motors (DC, Servo, and Stepper) operating simultaneously. A digital twin model monitors their behavior, detects faults in one motor, and redistributes load to maintain overall system performance.

# Inputs, Outputs, States, Parameters

This system models industrial multi-motor setups used in conveyor belts, robotics, and manufacturing lines, where fault detection and self-healing ensure continuous operation and prevent system failure.

| Inputs - Reference speed (ω_ref) - Voltage input to motors - Fault signal (disturbance) | Outputs - Motor speeds (ω₁, ω₂, ω₃) - Error signal (e = ω_ref − ω_actual) - Fault detection signal |
| --- | --- |
| State Variables - Angular velocity of each motor: | System Parameters - Motor constants (K) - Resistance (R) - Inductance (L) - Inertia (J) - Damping coefficient (B) - PID gains (Kp, Ki, Kd) |

# Degrees of Freedom (DOF)

The system has:  3 Degrees of Freedom

The system has 3 degrees of freedom since each motor rotates independently and requires one state variable (angular velocity) to describe its motion. There is no mechanical coupling between the motors; hence each contributes one independent DOF.

# Key Assumptions

- Motors are modelled as linear systems.

- Friction is viscous and proportional to velocity.

- Electrical dynamics are simplified into transfer functions.

- No mechanical coupling between motors (independent operation).

- Load sharing occurs only through control logic, not physical linkage.

- External disturbances affect only one motor at a time.

- System operates within stable region (no saturation effects).

# Coordinate System & Sign Convention

- Positive rotation direction is assumed clockwise.

- Speed is measured in rad/s.

- Error is defined as:

# Free Body Diagram

| For each motor: - Applied torque (T) - Friction torque (Bω) - Load torque - Inertia (J) - Change (dw/dt) | - Fault in motor = drop in Tmotor - T_{motor}Tmotor​ - Speed drops → ω decreases - Error increases → digital twin detects fault - Other motors increase torque → compensation |
| --- | --- |

# SYSTEM BEHAVIOR

- Under normal conditions, all motors track reference speed.

- When one motor experiences a fault (speed drop),

- The digital twin detects deviation using error threshold,

- Control system increases input to healthy motors,

- System maintains overall performance.

# Why This System is Dynamic

This is a dynamic system because:

- Output depends on time-varying inputs

- System involves inertia and energy storage

- Behaviour changes under disturbance